ARRL CONFIDENTIAL

Study of the Bands 902 MHz – 24.25 GHz

A Report to the ARRL Board of Directors

in Response to Minute 56 of the July 2002 Meeting

First Vice President, Joel Harrison, W5ZN

General Counsel Christopher Imlay, W3KD

Technical Relations Manager Paul Rinaldo, W4RI

May 22, 2003

Executive Summary

This study was commissioned by the Board of Directors as a timely and urgent evaluation of ARRL spectrum policies regarding domestic amateur allocations in the bands between 902 MHz through 24 GHz. It was prepared by the Technical Relations Office and the General Counsel per the Board’s directive, with contributions from the First Vice President, whose experience in on-air operation in these bands is extensive, and additional information provided by Director Frenaye. It is intended to be a wide-ranging analysis, including consideration of a relationship between the Amateur Radio Service and other services, regulatory impact, and possible alliances.

Amateur microwave bands do not exist in a vacuum. They are part of a complex set of international and domestic regulations. All are shared with other services or applications and most are allocated to the Amateur Radio Service on a secondary basis. The bands are situated in a part of the spectrum highly sought after for expanding commercial uses, both licensed and unlicensed. The regulatory environment is constantly changing and the pressure is increasing to find spectrum for new unlicensed devices. It is timely at this point to undertake this study, inasmuch as a series of conceptual changes and plans for user overlays and reallocation of spectrum, so as to accommodate new consumer and other commercial uses, are being implemented by FCC, with concurrent and resultant changes in threats and opportunities for radio amateurs.

This study discusses the regulatory environment, the allocations and uses of these bands, policy principles, and tentative recommendations. As was intended by Minute 56, this Study, presents and evaluates some zero-based options for deployment of Amateur uses of these challenged and challenging bands.

The authors would request that the Board review this without preexisting bias toward Amateur operating modes. In order to achieve success in this study, it is necessary to determine what defines success. What is it that we think Amateurs “ought to be doing” in these bands? What bands are good for one type of use but not for another? Is it necessary to have all types of Amateur operation duplicated in each of the bands? Can Amateur Radio be expanded in scope to take into consideration access to other information technologies? Can access to amateur spectrum be traded in exchange for access to non-amateur infrastructure with which to conduct Amateur Radio communications? These and other concepts challenge existing Board policy and business as usual. The authors believe that the Board deliberately intends to challenge past assumptions in the deployment of these bands, which has, until now, been something less than successful. Of course, neither has the defense of these unused bands been entirely successful either, but viewed objectively, it has been more successful than is justified or justifiable based on occupancy.  Protecting bands not used, or anticipated to be used, cannot be considered success for the long term. Deployment of the bands, even at the cost of access to substantial portions of that same spectrum, can, on the other hand, be considered success.

Table of Contents

Page

1.	INTRODUCTION									             1

1.1	Minute 56 of the July 2002 meeting of the ARRL Board of Directors			 1

1.2	Additional References										 1

1.3	Bands under Consideration in This Report							 2

2.	REGULATORY ENVIRONMENT							 3

2.1	Spectrum Policy Task Force, ET Docket 02-135						 3

2.2	Comprehensive Review of U.S. Spectrum Management with Broad Stakeholder

Involvement is Needed, GAO-03-277							 5

2.3	Additional 5-GHz Spectrum for Radio Local Area Networks, 				 6

WECA Petition RM-10371 and WRC-03 Agenda Item 1.5

2.4	Ultra-Wideband Transmission Systems, ET Docket No. 98-153

and ITU-R Task Group 1/8									 9

2.5	Additional Spectrum for Unlicensed Devices below 900 MHz and

in the 3-GHz Band, ET Docket No. 02-380							10

2.6	Amateur Primary Allocation at 2300-2305 MHz RM-10165, Microtrax

Petition RM-10165 and AeroAstro Petition RM-10166					11

2.7	3rd Generation Wireless Allocations, ET Docket No. 00-258				11

2.8	FCC OSP Working Paper No. 39 on Unlicensed Spectrum Devices				11

3.	ALLOCATIONS AND APPLICATIONS							12

3.1     	902-928 MHz Spectrum – Allocations and Uses						13

3.2  	1240-1300 MHz Spectrum – Allocations and Uses						14

3.3	2300-2450 MHz Spectrum – Allocations and Uses						15

3.4	3300-3500 MHz Spectrum – Allocations and Uses						16

3.5	5 GHz Spectrum – Allocations and Uses							17

3.6	10-10.5 GHz Spectrum – Allocations and Uses						19

3.7	 24-24.25 GHz Spectrum – Allocations and Uses						19

4.	BAND-BY-BAND ANALYSIS								20

4.1	902-928 MHz (33 cm Band)									20

4.2	1240-1300 MHz (23 cm Band)								23

4.3	2300-2305 MHz (Part of 13 cm Band)							25

4.4	2305-2310 MHz (Part of 13 cm Band)							26

4.5	2390-2400 MHz (Part of 13 cm Band)							26

4.6	2400-2450  (Part of 13 cm Band)								27

4.7	3300-3500 MHz (9 cm Band)								29

4.8	5650-5725 MHz (Part of 9 cm Band)							31

4.9	5725-5925 MHz (Part of 5 cm Band)							33

4.10	10-10.5 GHz (3 cm Band)									34

4.11	24-24.25 GHz (1.2 cm Band)								36

5.	DISCUSSION 										38

5.1	Propagation Properties									38

5.2	Strategic Partnering Initiatives								38

5.2.1	Partnering with Commercial, Licensed Services						39

5.2.2	Strategic Partnering with Public Safety and Disaster Relief Services			41

5.2.3	Strategic Partnering with Government Services					            42 5.2.4	Strategic Partnering with Unlicensed Devices and Systems				            43

5.3	Compatibility and Cooperation with Part 15 Applications				44

5.3.1	Compatibility											44

5.3.2	Confrontation											45

5.3.3	Interconnection with Part 15									45

5.3.5	Part 15 as a Recruiting Possibility								46

5.4	All-Band Microwave Radios								46

5.5	Special Circuits and Chips									47

5.5.1	Radio Interface Cards										47

5.5.2	Application Specific Integrated Circuits							48

5.5.3	Digital Voice Coder-Decoders								48

6.	GOALS AND RECOMMENDATIONS							50

6.1 	Goals for ARRL Spectrum Policy, 902 MHz – 24.25 GHz				50

6.2	Need to Overhaul the Band Planning Process						50

6.3	Band-by-Band Recommendations								50

6.3.1	902-928 MHz											50

6.3.2	1240-1300 MHz										50

6.3.3	2300-2305 MHz										50

6.3.4	2305-2310 MHz										51

6.3.5	2390-2400 MHz										51

6.3.6	2400-2450 MHz										51

6.3.7	3300-3500 MHz										51

6.3.8	5650-5725 MHz										51

6.3.9	5725-5925 MHz										51

6.3.10	10-10.5 GHz											51

6.3.11	24-24.25 GHz											51

6.4	Potential Replacement Spectrum								51

6.5	ARRL Incentives										52

6.6	Outreach Program										52

6.7	All-Band Radios										52

6.8	ARRL Continuing Education and Certification						52

Abbreviations and Acronyms

ACSSB	Amplitude Compandored Single Sideband

AMSAT	Radio Amateur Satellite Corporation

AMTOR	Amateur Teleprinting Over Radio

AP		Access Point (in local area network)

APCO		International. Association of Public-Safety Communications Officials.

ARIA		Amateur Radio Interference Assessment (ARRL)

ARR		Advanced Receiver Research

ARRL		American Radio Relay League–The National Association for Amateur Radio)

ARS		Amateur Radio Service (amateur service and amateur-satellite service)

ARSR		Aeronautical Route Surveillance Radar

ASIC		Application Specific Integrated Circuit

ATC		Air Traffic Control

ATV		Amateur Television

AVM		Automatic Vehicle Monitoring (FCC)

Bluetooth	A short-range radio standard

CEPT		European Conference of Post and Telecommunications Administrations

CPM		Conference Preparatory Meeting (ITU-R)

CPG		Conference Preparatory Group (CEPT)

CW		Morse code (on-off keyed carrier wave)

DEMI		Down East Microwave Inc

DFS		Dynamic Frequency Selection

DOD		Department of Defense

DOE		Department of Energy

DOT		Department of Transportation

DRM		Digital Radio Mondiale (HF digital broadcast standard)

DSN		Deep Space Network

DSRC		Dedicated Short Range Communications (Intelligent Transportation Systems)

ECP		European Common Proposal

EESS		Earth Exploration-Satellite Service

E.i.r.p.		Effective Isotropically Radiated Power (also EIRP)

EMC		Electromagnetic Compatibility

EME		Earth-Moon-Earth (also, “Moonbounce”)

EN		European Norm (standard)

ERC		European Radiocommunications Committee

ESA		European Space Agency

E-s		Earth-to-space

ETSI		European Telecommunications Standards Institute

EU		European Union

FAA		Federal Aviation Administration

FAX		Facsimile

FCC		Federal Communications Commission

FM		Frequency Modulation

FMTV		Frequency Modulated Television

FSTV		Fast Scan Television

FWA		Fixed Wireless Access

GAO		General Accounting Office, US Congress

GLONASS	Global Navigation Satellite System (Russia)

GNSS		Global Navigation Satellite System

GPS		Global Positioning System

HF		High Frequency (3-30 MHz)

HIPERLAN	High Performance Radio Local Area Network (Europe)

HSMM	High Speed Multimedia Working Group (ARRL)

IAC		Industry Advisory Council (ARRL)

IAP		Inter-American Proposal

IARU		International Amateur Radio Union

Icom		Icom Inc (Japan)

IEEE		Institute of Electrical and Electronics Engineers

ISM		Industrial, Scientific and Medical applications

ITS		Intelligent Transportation Systems

ITU		International Telecommunication Union

ITU-R		Radiocommunication Sector (ITU)

JARL		Japan Amateur Radio League

JTG		Joint Task Group

LMS		Location and Monitoring Service (FCC)

Neg-Reg	Negotiated Regulations

NASA		National Aeronautics and Space Administration

NOAA		National Oceanic and Atmospheric Administration

NOI		Notice of Inquiry

NPRM		Notice of Proposed Rule Making

NPSTC	National Public Safety Telecommunications Council

NSF		National Science Foundation

NTIA		National Telecommunications and Information Administration

OBRA		Omnibus Budget Reconciliation Act of 1993

OET		Office of Engineering and Technology (FCC)

OSCAR	Orbiting Satellite Carrying Amateur Radio

OSP		Office of Strategic Planning and Policy Analysis (FCC)

PAN		Personal Area Network

PCMCIA	Personal Computer Memory Card International Association (also PC Card)

PDA		Personal Data Assistant

PLC		Power Line Communications

R&O		Report and Order

RAC		Radio Amateurs of Canada

REC		Recommendation

RES		Resolution

RF		Radio Frequency

RFID		Radio Frequency Identification

RIC		Radio Interface Card

RLAN		Radio Local Area Network (also Wireless Local Area Network-WLAN)

RNSS		Radionavigation-Satellite Service

SAB		Service Ancillary to Broadcasting

SAP		Service Ancillary to Program making

SAR		Synthetic Aperture Radar

s-E		Space-to-Earth

SPTF		Spectrum Policy Task Force (FCC)

SS		Spread Spectrum

SSB		Single Sideband

TAC		Technology Advisory Council (FCC)

TG 1/8		ITU-R Task Group on Ultra-Wideband Compatibility

TTY		Teletypewriter

TPC		Transmitter Power Control

TVDL		Television Downlink

VHF		Very High Frequency (30-300 MHz)

UHF		Ultra High Frequency (300-3000 MHz)

U-NII		Unlicensed National Information Infrastructure

UWB		Ultra-wideband

VERON	Vereniging voor Experimenteel Radio Onderzoek in Nederland

WAS		Wireless Access System

WECA		Wireless Ethernet Compatibility Alliance (WECA)(now WiFi Alliance)

WiFi		Wireless Fidelity (radio local area network)

WRC		World Radiocommunication Conference