Why is the ARRL Concerned About the FCC’s NPRM on BPL?

In both its April 2003 Notice of Inquiry and its February 2004 Notice of Proposed Rule Making (NPRM) on Access Broadband over Power Line (BPL), the Federal Communications Commission (FCC) has affirmed that BPL system operators have an absolute obligation to resolve any harmful interference to licensed radio services. The NPRM proposes that BPL systems must incorporate adaptive interference mitigation techniques, including a shut-down feature to deactivate units found to cause harmful interference. BPL system operators also would be required to supply installation details to an industry-operated data base. Why, then, does the ARRL, the national association for Amateur Radio, believe that licensed radio services would be inadequately protected?

While the ARRL appreciates the FCC’s acknowledgment of the interference potential of BPL, these provisions of the NPRM are insufficient to reduce the probability of interference to a level that could reasonably be addressed on a case-by-case basis.

There are more than 680,000 Amateur Radio stations licensed by the FCC. These stations are located throughout the country and are operated from vehicles and portable locations as well as from homes and other fixed locations.

The FCC has underestimated the likelihood of interference to Amateur Radio stations from BPL systems and has ignored the practical problems that will arise when BPL service is disrupted by transmissions from Amateur Radio stations.

The NPRM warns BPL system operators “that amateur operations are likely to present a difficult challenge in the deployment of Access BPL in cases where amateurs use high gain outdoor antennas that are located near power lines.” There are more than 150,000 such stations spread across the United States at this time, and the number is increasing as a result of changes in FCC license requirements. Therefore, the probability that a BPL system will cause interference to amateur stations is quite high.

The NPRM is silent with respect to mobile operations that typically take place in close proximity to medium-voltage power lines. Approximately 70,000 amateurs have stations that operate in the BPL frequency range installed in their vehicles. ARRL’s field observations using typical amateur equipment have documented BPL interference to mobile stations located hundreds of meters from the interference source. The FCC’s proposals provide no practical way for mobile operators to seek mitigation of BPL interference they encounter in their normal travels on public thoroughfares.

Tests also have shown that a low-power signal from a legally operated Amateur Radio station can block the operation of a BPL system located hundreds of meters away. The NPRM makes no reference to this problem. It is unrealistic to expect that consumers will regard this as a fault of the BPL system; they will blame the nearby radio amateur, who will face hostility, harassment, and worse through no fault of his or her own. This will have a chilling effect on legitimate amateur operation.

BPL systems are not intended to radiate. Their radiation is simply the unavoidable byproduct of using an inappropriate medium, namely an unshielded and unbalanced wire, to carry a broadband signal. But the FCC proposes to apply the limits for intentional radiators to BPL systems. The FCC then compounds the error by permitting manufacturers – who would do their own testing for compliance – to make measurements close to the device under test, and then to use an inappropriate formula to extrapolate the measurements to the distance actually specified in the rules. The result is that a device may be shown to “pass” when in fact it emits as much as 100 times the permitted radiation level.

The limits for intentional radiators were intended to apply to short-duration and/or narrowband emissions. They are too high to be applied to long-duration, broadband emissions such as from BPL systems. If a BPL system operating at these limits is located anywhere near a radio receiver that is tuned to a frequency being used by the BPL system, including conventional shortwave radio receivers, field tests have shown repeatedly that interference is inevitable.

BPL systems degrade the usefulness of the narrow and unique portion of the radio spectrum that provides long-distance communication without the need for any infrastructure other than the ionosphere itself. Because it is unique, this part of the radio spectrum is used by government agencies and the military, as well as by radio amateurs, for a variety of purposes including aviation and maritime safety, national defense and homeland security.

What is the ARRL’s Proposed Solution?

1. Set the radiated emission limit at a lower level that offers reasonable protection to nearby radio receivers. Work is underway in the ITU Radiocommunication Sector to develop an appropriate limit; if a limit must be set before completion of this work, it must be set so as to preclude interference to typical amateur mobile and fixed stations.

2. Require testing and measurement of a BPL system by an independent laboratory to ensure compliance prior to the initiation of service. Eliminate loopholes that permit the inappropriate extrapolation of measurements.

3. Require that the marketers of BPL services give clear notice to consumers, prior to the signing of any contract, that licensed radio services have priority and that the delivery of BPL services therefore cannot be guaranteed. The contract must include a written acknowledgment by the consumer.

4. Establish performance standards for interference mitigation. To offer any real protection to licensed services, the rules must require that interference be resolved immediately – i.e., in real time – and on a 24-hour-per-day, 7-day-per-week basis. The industry-operated data base of BPL systems must be accessible to the public. There must be severe enforcement penalties for failure to keep the data base up to date, and for failure to resolve a complaint in real time.

_____